MAX78000 Secure Bootloader

In-Application Programming

with Python® User Guide

UGxxxx, Rev 3; 8/21

Abstract

This user guide details how update the end-user software application in the MAX78000 through the in-application programming, plus how to program the host code into the MAX32630FTHR board. Details on the MAX78000 secure bootloader can be found in the MAX78000 Secure Bootloader User Guide.

Python is a registered trademark of the Python Software Foundation.

Table of Contents

Introduction	4

System Requirements	5

Maxim Toolchain Installation	5

Environment Setup	5

Programming the MAX32630FTHR	6

Hardware Setup	8

In-Application Programming	9

Installing Microsoft Visual C++ Runtime	9

Installing OpenSSL Library	10

Compiling the Hello World Example in the Release Package with the Make Command	14

Generating AES Keys	14

Generating Bootloader Compatible Binary Using Maxim SDK Project	15

Converting the .bin File to the .msbl File Format	15

Programming Keys to the Bootloader	16

MSBL File Programming	16

Locking the SWD Interface	18

Unlocking the SWD Interface	18

Revision History	19

List of Figures

Figure 1. The MAX32630FTHR and MAX32625PICO board connection.	6

Figure 2. Serial port list.	6

Figure 3. CDC device driver warning.	6

Figure 4. MAX32630FTHR host reset button.	7

Figure 5. MAX32630FTHR host blinking LED.	7

Figure 6. Pin Connection between the MAX32630FTHR and MAX78000FTHR	8

Figure 7. Downloading firmware with the download_fw_over_host.exe	17

Figure 8. Bootloader Configuration File.	18

List of Tables

Table 1. Pin Connection between the MAX32630FTHR and  MAX78000FTHR	8

Introduction

This application note provides the instructions to program example host code into the MAX32630FTHR development platform.

The document also gives details related to hardware setup and application programming by using the MAX78000 bootloader and example host code.

Note that the screenshots may differ according to the software versions, but the steps will be same.

System Requirements

To program the MAX78000 bootloader code into the MAX78000FTHR, the minimum requirements are as follows:

Windows® PC

Windows 10, Windows 7

OpenSSL

Maxim Toolchain Software (more information, including download and installation instructions, is in this document)

MAX78000FTHR and micro-USB cable

MAX32630FTHR and micro-USB cable

MAX32625PICO evaluation kit (EV kit) and micro-USB cable

Test wires to connect the MAX78000FTHR and MAX32630FTHR

Maxim Toolchain Installation

To install the Maxim Toolchain to your PC, use the following steps:

- Download the Maxim SDK here.

- After downloading is complete, double-click   	MaximMicrosSDK.exe and use the default settings and select Next until finished.

Environment Setup

- Copy the release package content to your PC under the C:\ MAX78000_MSBL folder.

Programming the MAX32630FTHR

To program example host code into the MAX32630FTHR, use the following steps:

- Connect the grey 10-pin connector to the MAX32630FTHR and the MAX32625PICO board.

Connect the micro-USB cable to the MAX32625PICO and the PC.

Connect the micro-USB cable to the MAX32630FTHR and the PC.

Figure 1. The MAX32630FTHR and MAX32625PICO board connection.

Wait a few minutes for the Windows driver to install, then verify that it is installed correctly.

In the Windows 10 search box, type Control Panel (or for Windows 7, click Control Panel on the right side of the Start Menu). Either click Hardware and Sound, then Device Manager, or type Device Manager in the search box in the upper right.

If the drivers have correctly installed, you should see one port listed as mbed Serial Port for the MAX32625PICO. Note the COM port number for the USB serial device.

Figure 2. Serial port list.

If you see the following, then you will need to install the correct Windows driver:

Figure 3. CDC device driver warning.

Download the Arm Mbed® Windows serial port driver here.

For Windows 10, run mbedWinSerial_16466.exe by double-clicking it.

Drag and drop max32630fthr-host-vx.x.x.bin into the correct DAPLINK drive.

Press the reset button on the MAX32630FTHR, as shown in Figure 4.

Figure 4. MAX32630FTHR host reset button.

Verify that the LED on the MAX32630FTHR is blinking, as shown in Figure 5.

Figure 5. MAX32630FTHR host blinking LED.

Hardware Setup

Connect the MAX32630FTHR and MAX78000 with test wires according to Table 1. Pin connection between MAX32630FTHR and MAX78000FTHR is given in Figure 6.

Table 1. Pin Connection between the MAX32630FTHR and 
MAX78000FTHR

| Pin Function | MAX78000FTHR | MAX32630FTHR |
| --- | --- | --- |
| EBL GPIO | P0.5 | P5.4 |
| I2C1_SCL | P0.16 | P3.5 + 4.7K pullup |
| I2C1_SDA | P0.17 | P3.4 + 4.7K pullup |
| RST | RSTN | P5.6 |
| GND | GND | GND |

Figure 6. Pin Connection between the MAX32630FTHR and MAX78000FTHR

In-Application Programming

Installing Microsoft Visual C++ Runtime

To download and install Microsoft Visual C++ Runtime, use the following steps:

- Visit the download link here and click Download button.

- Select vcredist_x86.exe and continue by selecting Next.

Download and run the setup file.

Read and click if you agree the terms. Then select Install.

Close the installation application.

Installing OpenSSL Library

To download and install OpenSSL Library, use the following steps:

- Visit OpenSSL downloads page here.

Select Win32 OpenSSL v1.1.10L Light package.

Download and run the installer.

Read and click if you agree the terms. Then select Next.

Leave the destination location default and click Next.

Leave “The Windows system directory” selected and click Next.

Click install.

When the installation is completed, click finish.

Compiling the Hello World Example in the Release Package with the Make Command

To compile the Hello World with the make command, use the following steps:

1. Navigate to the correct directory consisting Hello World Example in the MinGW window with the following command:

cd " C:\MAX78000_MSBL \MAX78000_Hello_World”

2. Enter the following command in the MinGW window and wait several minutes for the command to complete:

make

3. After successful compiling, the Hello world binary image max78000.bin will be in the “C:\MAX78000_MSBL \MAX78000_Hello_World\build” directory.

Be sure that the correct linker file is used for generating the .bin file. A sample linker file, max78000_with_bl.ld, can be found under the Hello_World example folder.

4. If you want to rebuild, then enter these commands respectively:

make clean

make

Generating AES Keys

Generated AES keys can be used for encrypting bin files during msbl file generation and loading to bootloader. Customer can use their own method for generating keys, but file format should be similar to the sample max78000_sample_key.txt file.

1. Navigate to the correct directory consisting key generation script in the MinGW window with the following command:

cd "C:\MAX78000_MSBL”

2. Enter the following command in the MinGW window to generate a key:

./generate_key_vX.sh

3. Rename generated key.txt as max78000_key.txt file for using msbl file generation or bootloader key programming.

Generating Bootloader Compatible Binary Using Maxim SDK Project

Following steps should be applied for generating bootloader compatible binary using Maxim SDK project:

- Update your linker file:

  - Copy max78000_with_bl.ld file which is located under MAX78000_MSBL \MAX78000_Hello_World  to your Maxim SDK project.

- Update the makefile

  - Set the linker file variable (LINKERFILE) in makefile as below:

    - Remove following lines from makefile:

# Point this variable to a linker file to override the default file

LINKERFILE=$(CMSIS_ROOT)/Device/Maxim/$(TARGET_UC)/Source/GCC/$(LINKER)

    - Add following lines to makefile:

# Point this variable to a linker file to override the default file

LINKERFILE=$(TARGET_LC)_with_bl.ld

  - Add following lines to makefile under all target for generating the necessary bin file:

all:

arm-none-eabi-objcopy $(BUILD_DIR)/$(PROJECT).elf -O binary $(BUILD_DIR)/$(PROJECT).bin

- Build the project.

- .bin file is generated under build folder.

Converting the .bin File to the .msbl File Format

The .msbl file is generated automatically by using a .msbl generator.

1. Navigate to the directory consisting msbl generator and the .bin file to be converted in the MinGW window.

2. Enter the following command in the MinGW window window to convert the .bin (max78000.bin as an example) application to a .msbl file:

./msblGenWin32.exe max78000.bin MAX78000 8192 max78000_key.txt

3. “max78000.msbl” file will be generated

Programming Keys to the Bootloader

To program the keys to the MAX78000 by using the MAX78000 bootloader, use the following steps:

- At the command prompt or in the PowerShell window, enter the following commands, replacing COMxx with the correct USB serial device COM port found in section Programming the MAX32630FTHR at step 4.b.

download_fw_over_host.exe -k max78000_key.txt -p COMXX

- Note that the shared keys are sample one and they should be updated accordingly before programming keys.

MSBL File Programming

To flash the application to the MAX78000 by using the MAX78000 bootloader, use the following steps:

- At the command prompt or in the PowerShell window, enter the following commands, replacing COMxx with the correct USB serial device COM port found in section Programming the MAX32630FTHR at step 4.b.

download_fw_over_host.exe -f MAX78000_Hello_World.msbl -p COMXX

Figure 7. Downloading firmware with the download_fw_over_host.exe

The application has now been flashed to the chip. For advanced usage of flasher script, refer to the Firmware_downloader_usage.txt file.

Locking the SWD Interface

To lock the SWD Interface, use the following steps:

- At file max78000_bl_config.cfg: Change the swd_lock value to 1 and save the file.

Figure 8. Bootloader Configuration File.

- Run following command:

configure_bootloader.exe -f max78000_bl_config.cfg -p COMXX

- Disconnect the micro-USB cable.

- Reconnect the micro-USB cable. SWD is locked now.

Unlocking the SWD Interface

To unlock the SWD Interface, use the following steps:

  - At file max78000_bl_config.cfg: Change the swd_lock value to 0 and save the file.

  - Run the following command:

./configure_bootloader.exe -f max78000_bl_config.cfg -p COMXX

  - Disconnect the micro-USB cable.

  - Reconnect the micro-USB cable Now key and application are erased, SWD is unlocked, so a new key can be loaded.

Note that SWD can be locked and unlocked up to 4 times.

Revision History

| REVISION NUMBER | REVISION DATE | DESCRIPTION | PAGES CHANGED |
| --- | --- | --- | --- |
| 0 | 3/21 | Initial release | — |
| 1 | 3/21 | - Programming the MAX78000 with the Bootloader Firmware section added - Pin Connection between the MAX32630FTHR and - MAX78000FTHR Figure added - Programming Keys to the Bootloader Section added | 5, 8, 16 |
| 2 | 5/21 | - SWD Lock and Unlock Sections are Added | 16,17 |
| 3 | 8/21 | - Generating Bootloader Compatible Binary Using Maxim SDK Project Section added | 15 |

©2021 by Maxim Integrated Products, Inc. All rights reserved. Information in this publication concerning the devices, applications, or technology described is intended to suggest possible uses and may be superseded. MAXIM INTEGRATED PRODUCTS, INC. DOES NOT ASSUME LIABILITY FOR OR PROVIDE A REPRESENTATION OF ACCURACY OF THE INFORMATION, DEVICES, OR TECHNOLOGY DESCRIBED IN THIS DOCUMENT. MAXIM ALSO DOES NOT ASSUME LIABILITY FOR INTELLECTUAL PROPERTY INFRINGEMENT RELATED IN ANY MANNER TO USE OF INFORMATION, DEVICES, OR TECHNOLOGY DESCRIBED HEREIN OR OTHERWISE. The information contained within this document has been verified according to the general principles of electrical and mechanical engineering or registered trademarks of Maxim Integrated Products, Inc. All other product or service names are the property of their respective owners.